PowerHub  PMIC(STM32)

User Manual

| DATE | CONTENT | VERSION |
| --- | --- | --- |
| 2025-02-27 | Firmware update | 2.0 |
| 2025-10-16 | Adds input detection. | 2.1 |

Table of Contents

I、Power Enable Reg	2

1. LED Power Enable	2

2. USB Power Enable	2

3. Grove Red Power Enable	2

4. Grove Blue Power Enable	2

5. PWR485&PWRCAN Power Enable	3

6. VAMeter Power Enable	3

7. Charge Power Enable	3

II、USB Mode	3

III、PWR485&PWRCAN Config	3

1. Voltage（LSB&MSB）	4

2. Current Limit Value	4

3. Power Output Control	4

4. Power Direction Control	4

IV、Voltage&Current Monitor	4

V、Detect Status	5

VI、LED Color	5

VII、LED Brightness	6

VIII、Button	6

IX、Wakeup Source	6

1. nSTBY_WKUP	6

2. VIN_DET	6

3. VUSB_DET	7

X、RTC Time	7

XI、RTC Alarm	7

XII、Bootloader Version	8

XIII、Firmware Version	8

XIV、I2C Address	8

- I、Power Enable Reg

| Address | 0x00 - 0x06 |
| --- | --- |
| Access | W/R |
| Function | Enables or disables the LED power, USB power, Grove Red port power, Grove Blue port power, PWR485 & PWRCAN power, voltage/current meter power, and charging power. |

- LED Power Enable

Controls the power enable of the RGB LED. Write 1 to turn on, write 0 to turn off. The default state is off.

- USB Power Enable

Controls the power enable of the USB port. Write 1 to turn on, write 0 to turn off. The default state is off.

- Grove Red Power Enable

Controls the power enable of the Grove Red port. Write 1 to turn on, write 0 to turn off. The default state is off.

- Grove Blue Power Enable

Controls the power enable of the Grove Blue port. Write 1 to turn on, write 0 to turn off. The default state is off.

- PWR485&PWRCAN Power Enable

Controls the power enable of the PWR485 & PWRCAN port. Write 1 to turn on, write 0 to turn off. The default state is off.

- VAMeter Power Enable

Controls the power enable of the voltage/current meter. Write 1 to turn on, write 0 to turn off. The default state is off.

- Charge Power Enable

Controls the charging enable. Write 1 to turn on, write 0 to turn off. The default state is off.

- II、USB Mode

| Address | 0x10 |
| --- | --- |
| Access | W/R |
| Function | Controls the USB port mode, switching between device mode and host mode. |

- 0：Slave mode，USB->ESP32(default)

- 1：Host mode(USB TypeA)，ESP32->USB TypeA device

- 2：Host mode(USB TypeC)，ESP32->USB TypeC device

- III、PWR485&PWRCAN Config

| Address | 0x20 - 0x24 |
| --- | --- |
| Access | W/R |
| Function | Controls the SC8721 voltage, maximum current, power switch (physical switch), and power direction (output or input). |

- Voltage（LSB&MSB）

Configures the output voltage of the SC8721, range 3000 mV to 20000 mV, with a step size of 20 mV.

- Current Limit Value

Calculation formula: Iout limit (mA) = (Current Limit Value + 1) * 8 / 0.62

Example: To set the current limit to 1000 mA, Current Limit Value = 1000 * 0.62 / 8 - 1 = 76

Note: The total output power must not exceed 20 W. If the output voltage is set high, the output current register will be limited to a lower value.

- Power Output Control

Configures the SC8721 output using a MOSFET for physical switching. Write 1 to turn on the power output, write 0 to turn off. The default state is off. (Note: When the power direction is set to output, it will be automatically turned on.)

- Power Direction Control

Configures the input/output direction of the SC8721. Write 1 to set the direction to output, write 0 to set the direction to input. The default state is input.

- IV、Voltage&Current Monitor

| Address | 0x30 - 0x47 |
| --- | --- |
| Access | R |
| Function | Retrieves the voltage and current from the INA226 on each channel. |

- V、Detect Status

| Address | 0x50~0x51 |
| --- | --- |
| Access | R |
| Function | Retrieves the charging status and power input status. |

- 1、Charge Status
0：no charge

- 1：charging

- 2：error

- 2、VIN Status
0：no input

- 1：input present

- VI、LED Color

| Address | 0x60 - 0x7F |
| --- | --- |
| Access | W/R |
| Function | Controls the color of 8 LEDs. |

Updates the color of the corresponding LED by writing the corresponding RGB values.

- VII、LED Brightness

| Address | 0x80 - 0x87 |
| --- | --- |
| Access | W/R |
| Function | Controls the brightness of 8 LEDs. |

Updates the brightness of the corresponding LED by writing the corresponding RGB values.

- VIII、Button

| Address | 0xA0 |
| --- | --- |
| Access | R |
| Function | Retrieves the button status. |

- 0:press;

- 1:no press

- IX、Wakeup Source

| Address | 0xB0 - 0xB2 |
| --- | --- |
| Access | W/R |
| Function | Configures the wake-up source. |

- nSTBY_WKUP

Configures whether to enable RTC clock wake-up. Write 0 to disable, write 1 to enable. The default state is enabled.

- VIN_DET

Configures whether to enable VIN insertion wake-up. Write 0 to disable, write 1 to enable. The default state is enabled.

- VUSB_DET

Configures whether to enable VUSB insertion wake-up. Write 0 to disable, write 1 to enable. The default state is enabled.

Note: At least one of the three wake-up sources must be enabled. If all three wake-up sources are disabled, sleep will not be allowed.

- X、RTC Time

| Address | 0xC0 - 0xC6 |
| --- | --- |
| Access | W/R |
| Function | Configures the RTC clock. |

Sec: 0-59

Minute: 0-59

Hour: 0-23

Day: 1-31

Month: 1-12

Year: 0-168

Week Day:0x01, Sunday; 0x02, Monday; 0x04, Tuesday; 0x08, Wednesday;

0x10: Thursday; 0x20: Friday; 0x40: Saturday; 0x80: Sunday

- XI、RTC Alarm

| Address | 0xD0 - 0xD3 |
| --- | --- |
| Access | W/R |
| Function | Configures the RTC alarm. |

Minute: 0-59

Hour: 0-23

Day: 1-31

Alarm Enable: 1: Alarm Enable; 0: Alarm Disable

- XII、Bootloader Version

| Address | 0xFC |
| --- | --- |
| Access | R |
| Function | Retrieves the bootloader version number. |

- XIII、Firmware Version

| Address | 0xFE |
| --- | --- |
| Access | R |
| Function | Retrieves the firmware version number. |

- XIV、I2C Address

| Address | 0xFF |
| --- | --- |
| Access | W/R |
| Function | Sets the I2C address. |